CAN COMMUNICATION USING ESP32 AND MCP2515

Connections

Code

Tx:

//==================================================================================//

// BMS TRANSMITTER - ESP32 #1

// Sends dummy battery management system parameters

//==================================================================================//

#include <SPI.h>

#include <mcp2515.h>

MCP2515 mcp2515(5);

#define BMS_DATA_ID 0x100  // CAN ID for BMS data

#define ACK_ID 0x101       // CAN ID for acknowledgment

// BMS Parameters (dummy values that will change)

float voltage = 12.50;      // Voltage in V

float current = 2.50;       // Current in A

int charge = 85;            // Charge percentage

float temperature = 25.0;   // Temperature in °C

int cycles = 240;           // Charge cycles

float power = 31.25;        // Power in W

String status = "Excellent"; // Battery status

unsigned long lastSend = 0;

unsigned long messageCount = 0;

//==================================================================================//

void setup() {

Serial.begin(9600);  // Changed to 9600 to match your screenshot

delay(2000);

Serial.println("\n===========================================");

Serial.println("   BMS TRANSMITTER");

Serial.println("===========================================");

SPI.begin();

mcp2515.reset();

delay(100);

// Auto-detect crystal

MCP2515::ERROR result = mcp2515.setBitrate(CAN_500KBPS, MCP_8MHZ);

if (result != MCP2515::ERROR_OK) {

mcp2515.setBitrate(CAN_500KBPS, MCP_16MHZ);

}

mcp2515.setNormalMode();

Serial.println("✓ CAN initialized");

Serial.println("✓ Sending BMS data...");

Serial.println("===========================================\n");

delay(1000);

}

//==================================================================================//

void loop() {

if (millis() - lastSend >= 2000) {

updateBMSData();

sendBMSData();

lastSend = millis();

}

}

//==================================================================================//

void updateBMSData() {

// Simulate changing battery parameters

voltage += random(-10, 10) / 100.0;

if (voltage < 11.0) voltage = 11.0;

if (voltage > 13.5) voltage = 13.5;

current += random(-20, 20) / 100.0;

if (current < 0.5) current = 0.5;

if (current > 5.0) current = 5.0;

charge += random(-2, 2);

if (charge < 0) charge = 0;

if (charge > 100) charge = 100;

temperature += random(-5, 5) / 10.0;

if (temperature < 20.0) temperature = 20.0;

if (temperature > 35.0) temperature = 35.0;

cycles += random(0, 2);

if (cycles > 500) cycles = 240;

power = voltage * current;

// Update status based on charge

if (charge > 80) status = "Excellent";

else if (charge > 50) status = "Good";

else if (charge > 20) status = "Low";

else status = "Critical";

}

//==================================================================================//

void sendBMSData() {

messageCount++;

// Pack BMS data into CAN frames

// We'll send 4 separate CAN messages to fit all data

// Frame 1: Voltage, Current (8 bytes)

struct can_frame frame1;

frame1.can_id = BMS_DATA_ID;

frame1.can_dlc = 8;

int voltageInt = (int)(voltage * 100);    // 12.70V -> 1270

int currentInt = (int)(current * 100);    // 2.50A -> 250

frame1.data[0] = (voltageInt >> 8) & 0xFF;

frame1.data[1] = voltageInt & 0xFF;

frame1.data[2] = (currentInt >> 8) & 0xFF;

frame1.data[3] = currentInt & 0xFF;

frame1.data[4] = charge;

frame1.data[5] = (int)(temperature * 10) & 0xFF;

frame1.data[6] = (cycles >> 8) & 0xFF;

frame1.data[7] = cycles & 0xFF;

Serial.print("Transmitting: BAT:");

Serial.print(voltage, 2);

Serial.print(",");

Serial.print(current, 2);

Serial.print(",");

Serial.print(charge);

Serial.print(",");

Serial.print(temperature, 1);

Serial.print(",");

Serial.print(cycles);

if (mcp2515.sendMessage(&frame1) == MCP2515::ERROR_OK) {

Serial.println(" → SENT ✓");

} else {

Serial.println(" → FAILED ✗");

}

delay(50);

}

Rx:

//==================================================================================//

// BMS RECEIVER - ESP32 #2

// Receives and displays battery management system parameters

//==================================================================================//

#include <SPI.h>

#include <mcp2515.h>

MCP2515 mcp2515(5);

#define BMS_DATA_ID 0x100  // CAN ID for BMS data

#define ACK_ID 0x101       // CAN ID for acknowledgment

// BMS Parameters

float voltage = 0.0;

float current = 0.0;

int charge = 0;

float temperature = 0.0;

int cycles = 0;

float power = 0.0;

String status = "Unknown";

unsigned long lastReceived = 0;

unsigned long totalReceived = 0;

//==================================================================================//

void setup() {

Serial.begin(9600);  // Changed to 9600 to match your screenshot

delay(2000);

Serial.println("\n===========================================");

Serial.println("   BMS RECEIVER");

Serial.println("===========================================");

SPI.begin();

mcp2515.reset();

delay(100);

// Auto-detect crystal

MCP2515::ERROR result = mcp2515.setBitrate(CAN_500KBPS, MCP_8MHZ);

if (result != MCP2515::ERROR_OK) {

mcp2515.setBitrate(CAN_500KBPS, MCP_16MHZ);

}

mcp2515.setNormalMode();

Serial.println("✓ CAN initialized");

Serial.println("✓ Waiting for BMS data...");

Serial.println("===========================================\n");

delay(1000);

}

//==================================================================================//

void loop() {

struct can_frame rxMsg;

if (mcp2515.readMessage(&rxMsg) == MCP2515::ERROR_OK) {

if (rxMsg.can_id == BMS_DATA_ID) {

totalReceived++;

// Decode BMS data

int voltageInt = (rxMsg.data[0] << 8) | rxMsg.data[1];

voltage = voltageInt / 100.0;

int currentInt = (rxMsg.data[2] << 8) | rxMsg.data[3];

current = currentInt / 100.0;

charge = rxMsg.data[4];

temperature = rxMsg.data[5] / 10.0;

cycles = (rxMsg.data[6] << 8) | rxMsg.data[7];

power = voltage * current;

// Determine status based on charge

if (charge > 80) status = "Excellent";

else if (charge > 50) status = "Good";

else if (charge > 20) status = "Low";

else status = "Critical";

// Display battery status in the exact format you want

displayBatteryStatus();

lastReceived = millis();

}

}

// Show "waiting" message if no data received for 5 seconds

if (millis() - lastReceived > 5000 && totalReceived > 0) {

Serial.println("⚠ No data received for 5 seconds...");

lastReceived = millis();

}

delay(10);

}

//==================================================================================//

void displayBatteryStatus() {

Serial.println("=== BATTERY STATUS ===");

Serial.print("Voltage:    ");

Serial.print(voltage, 2);

Serial.println(" V");

Serial.print("Current:    ");

Serial.print(current, 2);

Serial.println(" A");

Serial.print("Charge:     ");

Serial.print(charge);

Serial.println(" %");

Serial.print("Temp:       ");

Serial.print(temperature, 1);

Serial.println(" °C");

Serial.print("Cycles:     ");

Serial.println(cycles);

Serial.print("Power:      ");

Serial.print(power, 2);

Serial.println(" W");

Serial.print("Status:     ");

Serial.println(status);

Serial.println("======================\n");

}

Output: